Motion Connect — The Fix

Why paid users don’t connect, and exactly how to fix it

Diagnosis proven by manual testing on the RB5009 — not a guess.

# 1. The Diagnosis (proven)

WHAT WORKS:  The router, hotspot, VLAN 30, DHCP, NAT, DNS and Starlink WAN are all healthy. A device with matching credentials logs in and gets full internet.

THE BUG:  Paid users never connect because of a CREDENTIAL MISMATCH. Each user is created on the router, but the password submitted at login does not match the password stored on the user, so the router rejects it with “invalid username or password.” Every auto-created user therefore sits at uptime=0s.

This was confirmed by hand on the router. When the stored password was set to a known value and the device was logged in with that same value, the session opened immediately and the phone loaded Cloudflare and BBC:

# 1. user existed with a KNOWN password

:put [/ip hotspot user get [find name=test1] password]   ->  test1

# 2. logged the device in by MAC (server-side)

/ip hotspot active login user=test1 password=test1 \

ip=192.168.30.233 mac-address=62:DD:36:3B:07:38

# 3. session opened and held

/ip hotspot active print

# USER    ADDRESS          UPTIME

0 test1   192.168.30.233   2m32s     <-- SUCCESS

# 4. phone loaded 1.1.1.1 and bbc.com  -> full internet

Conclusion: nothing on the router needs rebuilding. The fix is entirely in how the backend authorises a device after payment.

# 2. The Fix — MAC-based server-side login

Stop relying on the student’s browser to submit a username/password form. That approach caused two problems: the credential mismatch above, and a browser “this form is not secure” warning that broke the redirect. Instead, after payment confirms, the BACKEND logs the device in directly by MAC — the exact operation proven above.

## Corrected flow

Student connects to Student SSID

|

Captive portal opens (Motion Connect)

|

Student picks package + enters MoMo number

|

Backend -> Hubtel  (charge MoMo)

|

Student approves on phone

|

Hubtel -> webhook -> Backend   (payment confirmed)

|

*** THE FIX HAPPENS HERE ***

Backend logs the device in BY MAC on the router:

/ip hotspot active login user=.. password=.. ip=.. mac-address=..

|

Phone is online. No browser form. No password typing. No mismatch.

THE ONE RULE:  Use a single password value for both creating the user and logging it in. Never generate a second password anywhere in the flow. That single discipline eliminates the mismatch.

## The sequence to implement

// Run AFTER Hubtel confirms payment (in the webhook handler / a queue worker)

1. isActive(mac)            // already online? -> done (idempotent)

2. findHost(mac) -> ip      // device must be on the SSID; get its current IP

3. upsertUser({             // create OR update the user

username, password,    //   password stored here MUST equal step 4's

profile, mac, bytes    //   bytes = data cap (limit-bytes-total)

})

4. loginDevice({            // THE KEY CALL - server-side MAC login

username, password,    //   SAME password as step 3

ip, mac

})

// The single rule that fixes the bug:

//   the password on the user  ==  the password sent to login

//   (use ONE variable for both; never generate a second one)

# 3. Code — drop-in module

Two files accompany this brief: mikrotik-login.js (the RouterOS REST API helper with grantAccess()) and router-poll.rsc (the recommended no-inbound approach). Call grantAccess() from the Hubtel webhook handler once payment is confirmed:

import { grantAccess } from './mikrotik-login.js';

// inside the Hubtel callback, after verifying success + amount:

const result = await grantAccess({

username: session.username,   // the user created for this payment

password: session.password,   // SAME value stored on the router user

profile:  pkg.profile,        // e.g. "weekly"

mac:      session.mac,        // captured by the portal at checkout

bytes:    pkg.bytes,          // data cap

});

if (!result.ok && result.reason === 'device-not-on-network') {

// student's phone dropped off the SSID; retry on next poll or

// ask them to reconnect. Do NOT mark the payment failed.

}

# 4. Hosting reality (important)

CONSTRAINT:  The router (192.168.20.1) is only reachable from the LAN. A Vercel serverless function on the public internet CANNOT call it directly. The login must be triggered from something that can reach the router.

Two valid patterns:

Option B — router polls (recommended for Starlink). The router runs a scheduler that polls Vercel every ~20s for paid sessions and performs the MAC login locally. Nothing inbound is opened on the router; it survives the dynamic Starlink IP. Use the included router-poll.rsc. The developer builds two small endpoints:

- GET /api/pending-authorizations — returns paid-not-activated sessions (with the X-Router-Key header check).

- POST /api/mark-activated — marks the returned ids as done so they aren’t handed out twice.

Option A — LAN agent. Run mikrotik-login.js on a small always-on machine on-site that can reach 192.168.20.1. Vercel handles Hubtel + UI and hands paid sessions to that agent.

EITHER WAY:  The password returned to the router MUST equal the password stored on the hotspot user. One value, used in both places.

# 5. Verify the fix

- Deploy the change. Make a GHS 1 test payment on a phone connected to the Student SSID.

- Within ~20s (Option B) or immediately (Option A), on the router run /ip hotspot active print — the paid user should appear with uptime > 0.

- On the phone, open http://1.1.1.1 then a normal site — both should load.

- Confirm the user shows real traffic: bytes-in / bytes-out should be non-zero (previously always 0).

Cleanup after testing: remove the stale zero-uptime test users so they don’t clutter the list.

/ip hotspot user remove [find where uptime="0s" and name~"mc-"]

Summary: the infrastructure is proven working. The only change needed is server-side, MAC-based login after payment, using one password value for both creating the user and logging it in. Keep the API user password and Hubtel secrets in environment variables.